GLOVE STANDARDS

To conform with European legislation, all gloves supplied as personal protective equipment must comply with the Personal Protective Equipment Directive [implemented into UK law by The Personal Protective Equipment (EC Directive) Regulations 1992] and be CE marked.

Products such as gardening gloves, of “simple design”, protecting from minor risk of injury can be self-certified by the manufacturer or importer.  Gloves worn to protect the user from potentially life threatening risks must be independently tested by an approved test house to an appropriate European Standard. A summary of the main glove standards is given below:

BS EN 420	General requirements for all gloves

(Sizing, product and packaging information and marking, etc.)

BS EN 374-2	Resistance to penetration by micro-organisms

BS EN 374-3	Resistance to chemical hazards

BS EN 381	Chainsaw hazards

BS EN 388	Protect against mechanical hazards

BS EN 407	Protect against heat and fire

BS EN 421	Protect against ionising radiation

BS EN 511	Protect against low temperature

BS EN 659	Firefighters’ gloves

One of the important benefits of the standards is marking, which reduces the possibility of gloves being used incorrectly.

Gloves in the “simple design” category (protecting against minor risk) will have packaging marked with the words “For minimal risks only”.  Other gloves will have more information on the packaging, including:

The CE mark – showing it conforms to the appropriate standard, with the approved body identification number

A pictogram indicating the gloves protective properties

A series of numbers accompanying the pictogram, indicating its performance in the various tests applicable to that standard.

GLOVE STANDARDS AND ASSOCIATED MARKINGS

| STANDARD | PICTOGRAM | DESCRIPTION | RATING |
| --- | --- | --- | --- |
| BS EN 374 – 2 Micro-organisms |  | Resistance to penetration by micro-organisms | 1-6 |
| BS EN 374 – 3 Chemical Hazards |  | Resistance to chemical permeation | 1-3 |
| BS EN 388 Mechanical Hazards |  | a) Resistance to abrasion b) Blade cut resistance c) Tear resistance d) Puncture resistance | 0-4 0-5 0-4 0-4 |
|  |  | Impact Cut | Pass/Fail |
|  |  | Static Electricity | Pass/Fail |
| BS EN 407 Thermal Hazards |  | a) Burning behaviour b) Contact heat c) Convective heat d) Radiant heat e) Small splashes of molten metal f) Large splashes of molten metal | 0-4 0-4 0-4 0-4 0-4 0-4 x = test NA |
| BS EN 511 Protection from Cold |  | a) Resistance to convective cold b) Resistance to contact cold c) Permeability to water | 0-4 0-4 0-1 |
| BS EN 421 Radioactive Contamination |  | Resistance to ionising radiation (the amount of lead in the glove is marked on it) |  |

Note: For ratings, the higher the number the higher the level of performance